The impact of production interruptions on kitting, an analytical study

Eline De Cuypere

Supervisor(s): Dieter Fiems, Herwig Bruneel

# Introduction

Efficient transport of materials between the stages of the production process is key in the minimization of production costs. The kitting process is an attempt at achieving efficient transport and thus reducing costs. In kitting the necessary parts for assembly are collected into a specific container, a kit prior to arriving at the assembly line [1].  This is in contrast to a system where parts arrive at the assembly line in containers of equal parts.

We analyze the kitting process as a two-queue model in a stochastic assembly system. In such system, the parts arrive at a buffer and “wait” until they are collected into a kit. Each of the two types of parts are necessary to compose one kit.

# Methodology

We model the kitting process as a (Continuous Time) Markov Chain with a multidimensional state space which approximates all possible inventory levels. While the state space of the transition matrix is large, the number of possible state transitions from any specific state is limited. This means that most of the entries in the transition matrix are zero, i.e. a sparse matrix. Techniques to define and solve sparse matrices (e.g. the generalized minimal residual method) are applied.

Figure 1 illustrates a numerical example of the probability that the second part buffer is full for three models.  The first model does not consider temporary interruptions in the production whereas the second model accounts for interruptions for part one and the third model accounts for interruptions in both.

Figure 1. Probability that the second part buffer full is.

We assume a finite-buffer-capacity for both buffers as shown on the x-axis.

Temporary interruptions in the production of one part have a significant impact on the buffer of the other part (model 1 & 2).   However, adding a production interruption component at part two does not significantly impact the probability (model 3).

# Conclusion

The sparse matrix technique is a valuable queuing theoretic numerical approach for estimating the performance of a kitting process in terms of solution speed and accuracy.

References

- [1] Y.A. Bozer and L.F. McGinnis, Kitting versus line stocking: A conceptual framework and a descriptive model, International Journal of Production Economics, 25:1-19, 1992.