The impact of production interruptions on kitting, an analytical study

Eline De Cuypere

Supervisor(s): Dieter Fiems, Herwig Bruneel

# Introduction

The efficient transport of materials between the different stages of the production process is a key issue for overall production cost minimization. Therefore the kitting process is introduced. Instead of delivering parts at the assembly line in containers of equal parts, in kitting prior to arriving at an assembly unit the necessary parts are collected into a specific container, referred to as “kit” [1].

This article analyzes the kitting process as a two-queue model in an stochastic assembly system. Each of the two types of parts are necessary to compose one kit. The parts arrive at the part buffers and “wait” there till they are collected into a kit.

# Methodology

We model the kitting process as a (Continuous Time) Markov Chain with a multidimensional state space that roughly includes all possible inventory levels. While the state space of the transition matrix is large, the number of possible state transitions from any specific state is limited. This means that most of the entries in the transition matrix are zero, i.e. a sparse matrix. Techniques to define and solve sparse matrices (e.g. the generalized minimal residual method) are applied.

Figure 1 illustrates a numerical example of the probability that the second part buffer full is for the three models: the first model doesn’t consider temporary interruptions in the production, the second models interruptions for part one and the last model for both parts. We assume a finite-buffer-capacity for both buffers showed on the x-axis.

Figure 1. Probability that the second part buffer full is.

Temporary interruptions in the production of one part have a significant impact on the buffer of the other part (model 1 & 2). Then, if we “add” a production interruption component at part two, this doesn’t have a significant impact on the probability (model 3).

# Conclusion

The sparse matrix techniques are a valuable queuing theoretic numerical approach in terms of solution speed and accuracy to estimate the performance of the kitting process.

References

- [1] Y.A. Bozer and L.F. McGinnis, Kitting versus line stocking: A conceptual framework and a descriptive model, International Journal of Production Economics, 25:1-19, 1992.